# MATERIAL SAFETY DATA SHEET

| CHEMTRON INCORPORATED | EMERGENCY INFORMATION: 800-424-9300 |
| --- | --- |
| 7361 H LOCKPORT PLACE | CHEMTREC |
| LORTON, VIRGINIA 22079 |  |
| INFORMATION: 703-550-7772 | DATE OF ISSUE: 03-07-03 |

| SECTION 1 IDENTIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| PRODUCT NAME: LIME OUT |  |  |  |  |
| PRODUCT TYPE: LIME SCALE REMOVER |  |  |  |  |

| SECTION 2 HAZARDOUS INGREDIENTS |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | CAS NUMBER | % | TWA | TLV |
| 1. PHOSPHORIC ACID | 7664-38-2 | 35.5 | 2MG/m2 | 1mg/m2 |
| 2. OXIRANE, METHYL-POLYMER, | 9003-11-6 | <2 | NONE ESTABLISHED |  |
| TWA = TIME WEIGHED AVERAGE, TLV = THRESHOLD LIMIT VALUE, C = CEILING LIMIT |  |  |  |  |

| SECTION 3 PHYSICAL DATA |  |
| --- | --- |
| APPEARANCE AND ODOR: | Clear green liquid. |
| SOLUBILITY IN WATER: | Complete |
| PH: | 2-3 |
| BOILING POINT: | 220 degrees Fahrenheit |
| SPECIFIC GRAVITY: | 1.207-1.215 |
| PERCENT VOLATILE: | 62.2 |

| SECTION 4 FIRE AND EXPLOSION HAZARD DATA |  |
| --- | --- |
| SPECIAL HAZARDS: | Non-Flammable however, thermal decomposition emits toxic fumes of oxides of phosphorus. Flammable hydrogen gas may be released upon contact with soft metals such as aluminum or zinc. |
| FIREFIGHTING METHODS: | Due to the corrosive nature of this product use full protective clothing and SCNBA apparatus. All recognized methods acceptable |
| EXTINGUISHING MEDIA: | All recognized methods acceptable. |
| UNUSUAL EXPLOSION AND FIRE HAZARDS: | Non-flammable. |
| HAZARDOUS POLYMERIZATION WILL OCCUR |  |

| SECTION 5 REACTIVITY DATA |  |
| --- | --- |
| STABILITY: | Stable |
| CONDITIONS TO AVOID: | Do not store at elevated temperatures |
| INCOMPATIBILITY: | Do not mix with alkali or oxidizing agents. |
| HAZARDOUS DECOMPOSITION PRODUCTS: | Oxides of Phosphorus & Carbon |

| SECTION 6 SPILL OR LEAK PROCEDURES |  |
| --- | --- |
| CLEANUP: | For small spills mop up and rinse the area with water. For large spills dike area and avoid run off into sewers and surface waters. Soak up in an absorbent material and place into a DOT approved waste drum. |
| WASTE DISPOSAL: | Consult current local regulations for disposal. Dispose of in accordance with all local, state, and federal laws. |

| SECTION 7 HEALTH HAZARD DATA |  |
| --- | --- |
| EFFECTS OF OVEREXPOSURE |  |
| SKIN: | The product is a severe irritant to the skin and corrosive to tissue. |
| EYES: | Redness and severe burning to the eyes. Product may cause permanent damage to eyes. |
| INHALATION: | Irritant to the airways and lungs, damage depends upon the amount and duration of exposure. |
| INGESTION: | The product may cause irritation of the mouth, throat and stomach. The product is mildly corrosive and a mild skin irritant. Single dose toxicity is minimal. |

| SECTION 8 FIRST AID |  |
| --- | --- |
| EYES: | Immediately flush with cool running water. Remove contact lenses. Continue flushing with running water for 15 minutes, holding eyelids open to ensure rinsing of the entire eye. |
| SKIN: | Immediately flush skin with plenty of cool running water for 15 minutes. Remove contaminated clothing if necessary and launder before reuse. |
| IF SWALLOWED: | Rinse mouth at once, then drink two large glasses of water. DO NOT INDUCE VOMITING. Never give anything to mouth of an unconscious person. Seek medical attention immediately. |
| INHALATION: | Remove to fresh air. |

| SECTION 9 SPECIAL PROTECTION DATA |  |
| --- | --- |
| RESPIRATORY: | Avoid mist, wear respirator if necessary |
| SKIN: | Rubber gloves if exposure is possible |
| EYES: | Avoid contact, use goggles if exposure is possible goggles |
| OTHER: | NONE ESTABLISHED |
| VENTILATION: | Maintain adequate ventilation |

| SECTION 10 PRECAUTIONS AND SPECIAL INFORMATION |  |
| --- | --- |
| Storage and Handling: | Keep product cool and dry. To insure maximum shelf life store at normal room temperature. |
| DOT Classification: | Corrosive liquid N.O.S. (Phosphoric Acid), Corrosive Material, UN 1760, Corrosive, PGII, 8 |
| This is an industrial cleaning compound so do not mix with anything but water and keep out of reach of children. |  |
| The above information is believed to be correct with respect to the manufactured formula. As new data is developed standards and regulations change and, of course, use and handling are beyond our control. No warranty, expressed or implied, is made as to the completeness or continuing accuracy of this information. |  |

# Home